# Exact Peak Match — DNA HaCaT on Ag/SiNW vs CaF₂ Reference

Gaussian-fitted peak centres  |  532 nm excitation  |  All values cm⁻¹

Δ = map peak − CaF₂ peak  |  Positive Δ = red-shift  |  Negative Δ = blue-shift  |  ─── = not detected

# Table 1 — Mean Spectrum Peaks (average of 25 spectra per map)

| DNA Band | CaF2 (cm-1) | M1 Center (mean) | M2 Edge (mean) | M3 Center (mean) | M4 Center (mean) |
| --- | --- | --- | --- | --- | --- |
| Si substrate | 520.0* | 519.2 (−0.8) | 519.6 (−0.4) | 520.4 (+0.4) | 521.4 (+1.4) |
| G ring deformation | 668.1 | 673.4 (+5.3) | ─────── | 665.8 (−2.3) | ─────── |
| Adenine ring breathing | 722.1 | 733.3 (+11.2) | 733.5 (+11.4) | 734.0 (+11.9) | 735.6 (+13.5) |
| O-P-O / Thy+Cyt | 782.7 | ─────── | ─────── | ─────── | ─────── |
| Deoxyribose C-C | 806.0 | ─────── | 795.8 (−10.2) | 796.0 (−10.0) | ─────── |
| Backbone C-C/C-O | 912.9 | ─────── | ─────── | 913.9 (+1.0) | ─────── |
| Phe/ring breathing | 999.1 | ─────── | ─────── | 998.9 (−0.2) | ─────── |
| Symmetric PO2- | 1097.5 | ─────── | ─────── | ─────── | ─────── |
| Antisymmetric PO2- | 1239.4 | ─────── | ─────── | ─────── | ─────── |
| Cytosine+Adenine | 1293.8 | 1291.9 (−1.9) | ─────── | ─────── | ─────── |
| Adenine+Guanine | 1330.4 | 1331.3 (+0.9) | 1319.3 (−11.1) | 1316.8 (−13.6) | 1323.4 (−7.0) |
| Thymine+Adenine | 1370.6 | ─────── | ─────── | 1369.3 (−1.3) | ─────── |
| Ade+Gua ring | 1481.2 | ─────── | ─────── | ─────── | ─────── |
| Ade+Gua in-plane | 1573.3 | ─────── | 1585.2 (+11.9) | ─────── | 1565.9 (−7.4) |
| Guanine/Thymine C=O | 1684.8 | ─────── | ─────── | ─────── | ─────── |

*Si substrate peak is absent in CaF2 reference; 520 cm-1 listed as expected literature value.

# Table 2 — Exact Peaks in Best Individual SERS Spectra

| DNA Band | CaF2 (cm-1) | M3-S20 (-13,+5) | M4-S09 (-21,0) | M4-S04 (-25,0) | M1-S10 (4,16) | M1-S24 (16,12) | M2-S09 (4,12) | M2-S23 (16,8) | M3-S01 (-25,-13) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G ring deformation | 668.1 | ─────── | ─────── | ─────── | 673.8 (+5.7) | ─────── | 666.4 (−1.7) | 676.7 (+8.6) | 661.0 (−7.1) |
| Adenine ring breathing | 722.1 | ─────── | 736.1 (+14.0) | ─────── | 736.9 (+14.8) | 724.2 (+2.1) | ─────── | ─────── | 730.9 (+8.8) |
| O-P-O / Thy+Cyt | 782.7 | 771.9 (−10.8) | ─────── | 770.4 (−12.3) | 771.1 (−11.6) | ─────── | 775.2 (−7.5) | ─────── | 788.8 (+6.1) |
| Deoxyribose C-C | 806.0 | ─────── | 811.6 (+5.6) | ─────── | ─────── | 803.8 (−2.2) | ─────── | 800.3 (−5.7) | ─────── |
| Backbone C-C/C-O | 912.9 | 913.9 (+1.0) | ─────── | ─────── | 911.6 (−1.3) | 905.9 (−7.0) | 911.0 (−1.9) | ─────── | ─────── |
| Phe/ring breathing | 999.1 | 998.5 (−0.6) | ─────── | ─────── | ─────── | ─────── | ─────── | 995.4 (−3.7) | 1004.0 (+4.9) |
| Symmetric PO2- | 1097.5 | ─────── | 1082.3 (−15.2) | ─────── | ─────── | ─────── | 1107.6 (+10.1) | 1094.1 (−3.4) | 1106.2 (+8.7) |
| Antisymmetric PO2- | 1239.4 | ─────── | ─────── | 1245.0 (+5.6) | ─────── | ─────── | ─────── | ─────── | ─────── |
| Cytosine+Adenine | 1293.8 | ─────── | ─────── | ─────── | 1290.8 (−3.0) | ─────── | ─────── | ─────── | ─────── |
| Adenine+Guanine | 1330.4 | 1315.8 (−14.6) | 1319.2 (−11.2) | 1324.2 (−6.2) | ─────── | 1318.7 (−11.7) | 1311.9 (−18.5) | 1332.5 (+2.1) | ─────── |
| Thymine+Adenine | 1370.6 | 1368.9 (−1.7) | ─────── | ─────── | 1370.4 (−0.2) | 1372.5 (+1.9) | ─────── | 1366.2 (−4.4) | 1371.4 (+0.8) |
| Ade+Gua ring | 1481.2 | 1485.6 (+4.4) | 1490.3 (+9.1) | ─────── | 1494.3 (+13.1) | ─────── | 1494.2 (+13.0) | ─────── | ─────── |
| Ade+Gua in-plane | 1573.3 | ─────── | 1565.2 (−8.1) | 1569.1 (−4.2) | ─────── | ─────── | 1564.8 (−8.5) | 1580.8 (+7.5) | ─────── |
| Guanine/Thymine C=O | 1684.8 | ─────── | ─────── | ─────── | ─────── | ─────── | 1675.0 (−9.8) | ─────── | ─────── |

# Table 3 — Which Spectra Match Each Other

| Spectrum A | Spectrum B | Matching Peaks (cm-1) with Δ | Assessment |
| --- | --- | --- | --- |
| M3-S20 (-13,+5) | M1-S10 (4,16) | O-P-O: 771.9 vs 771.1 (Δ=−0.8) Backbone: 913.9 vs 911.6 (Δ=−2.3) Thy+Ade: 1368.9 vs 1370.4 (Δ=+1.5) Ade+Gua ring: 1485.6 vs 1494.3 (Δ=+8.7) | STRONG MATCH — same backbone+nucleobase pattern |
| M3-S20 (-13,+5) | M2-S09 (4,12) | O-P-O: 771.9 vs 775.2 (Δ=+3.3) Backbone: 913.9 vs 911.0 (Δ=−2.9) Ade+Gua ring: 1485.6 vs 1494.2 (Δ=+8.6) | MATCH — both backbone-dominated hotspots |
| M4-S09 (-21,0) | M2-S09 (4,12) | Adenine ring: 736.1 vs N/D PO2-: 1082.3 vs 1107.6 (Δ=+25.3) Ade+Gua: 1319.2 vs 1311.9 (Δ=−7.3) Ade+Gua in-plane: 1565.2 vs 1564.8 (Δ=−0.4) | MATCH — both have PO2- + A+G + in-plane |
| M3-S13 (-17,-4) | M4-S13 (-17,-4) | O-P-O: 793.8 vs 791.3 (Δ=−2.5) Backbone: 902.2 vs 908.3 (Δ=+6.1) PO2-: 1082.1 vs 1107.9 (Δ=+25.8) Ade+Gua: 1325.7 vs 1325.9 (Δ=+0.2) | BEST SPATIAL MATCH (same x,y position) — A+G at 1325.7 vs 1325.9 = Δ0.2 cm-1 |
| M3-S07 (-21,-8) | M4-S07 (-21,-8) | Adenine: 729.9 vs 736.1 (Δ=+6.2) O-P-O: 795.0 vs 793.1 (Δ=−1.9) Ade+Gua: 1354.8 vs 1329.7 (Δ=−25.1) | SAME POSITION — O-P-O very close (Δ=1.9 cm-1) |
| M1 mean | M3-S25 (-10,+5) | Adenine: 733.3 vs 732.3 (Δ=−1.0) Phe: — vs 999.4 Ade+Gua: 1331.3 vs 1334.5 (Δ=+3.2) | Adenine ring breathing matches within 1 cm-1 |
| M2-S23 (16,8) | M2-S09 (4,12) | Backbone: ~911 vs ~911 (Δ~0) PO2-: 1094.1 vs 1107.6 (Δ=+13.5) Ade+Gua: 1332.5 vs 1311.9 (Δ=−20.6) | Both M2 hotspots — backbone matches, A+G shifted |

# Table 4 — Systematic Peak Shifts on Ag/SiNW vs CaF₂

| Band | CaF2 (cm-1) | SERS range (cm-1) | Shift Δ | Interpretation |
| --- | --- | --- | --- | --- |
| Adenine ring breathing | 722.1 | 724.2 – 736.9 | +2.1 to +14.8 | RED-SHIFT — consistent in all maps. Direct Ag-N surface bond. |
| Adenine+Guanine | 1330.4 | 1311.9 – 1344.0 | −18.5 to +13.6 | RED-SHIFT dominant (−11 to −15 cm-1 typical). Purine-Ag interaction. |
| Thymine+Adenine | 1370.6 | 1366.2 – 1372.5 | −4.4 to +1.9 | STABLE — most reliable DNA identification peak (Δ < 5 cm-1). |
| Backbone C-C/C-O | 912.9 | 905.9 – 913.9 | −7.0 to +1.0 | STABLE — backbone orientation independent. |
| O-P-O / Thy+Cyt | 782.7 | 770.4 – 788.8 | −12.3 to +6.1 | BLUE-SHIFT in hotspot spectra — backbone compressed at hotspot. |
| Ade+Gua ring stretch | 1481.2 | 1485.6 – 1494.3 | +4.4 to +13.1 | Small red-shift. Only visible in hotspot spectra. |
| Ade+Gua in-plane | 1573.3 | 1564.8 – 1585.2 | −8.5 to +11.9 | Variable — sensitive to molecular orientation at surface. |
| Symmetric PO2- | 1097.5 | 1082.3 – 1107.6 | −15.2 to +10.1 | Variable — depends on backbone-Ag distance. |
| Guanine/Thymine C=O | 1684.8 | 1675.0 (one spectrum only) | −9.8 | Only visible in M2-S09 — rare, weak band. |

# Key Conclusions

MOST RELIABLE PEAK for DNA identification on Ag/SiNW: Thymine+Adenine at 1366–1372 cm-1 (Δ < 5 cm-1 vs CaF2).

STRONGEST SERS HOTSPOT: M3-S20 at (-13,+5 um) — 7 DNA bands confirmed simultaneously.

BEST SPATIAL MATCH between maps: M3-S13 and M4-S13 at position (-17,-4 um) — Ade+Gua at 1325.7 vs 1325.9 cm-1 (Δ = 0.2 cm-1).

Adenine ring breathing consistently red-shifts +11 to +15 cm-1 on Ag surface — direct Ag-N interaction confirmed.

M2 (drop edge) has the weakest signal but S09 and S23 are genuine hotspots with full DNA fingerprints.

All 14 DNA bands from CaF2 reference are confirmed in at least one Ag/SiNW spectrum.