# Acacia EVs vs NVs - PCA / LDA figure + CH-stretching mean +/- SD figure

How to use:

1. In PyCharm: File > New > Python File, name it Acacia_PCA_LDA_Figure.py.

2. Select all the code below (from the first "# ===" line to the last line), copy and paste it into that file.

3. In the PyCharm Terminal, install the packages once:  pip install numpy scipy pandas openpyxl matplotlib scikit-learn

4. Run the file. Data are read from C:\Users\Hira Aman\Downloads\CODE 005 and the figures Fig_PCA_LDA.png / .tiff and Fig_MeanSD_CH.png / .tiff and the band table MeanSD_CH_bands.csv are saved in CODE 005\RESULTS.

------------------------------------------------------------

# =============================================================================

# ACACIA EVs vs NVs  |  SERS on Ag/SiNWs  |  PCA scores + LDA canonical variable

#

#   (a) PCA scores, fingerprint region (250-1790 cm-1)

#   (b) canonical variable 1 (PCA-LDA, 5 PCs), fingerprint region

#   (c) PCA scores, CH-stretching region (2760-3095 cm-1)

#   (d) canonical variable 1 (PCA-LDA, 5 PCs), CH-stretching region

# and a second figure: mean +/- SD (sample SD, n - 1) CH-stretching spectra of EVs

# and NVs (2760-3095 cm-1) with band positions and assignments.

#

# Same data, pre-processing and models as Acacia_EV_NV_Analysis.py (Figure 4,

# panels a, b, d, e):

#   cosmic-ray removal (modified z-score, threshold 7) -> 2 cm-1 grid ->

#   Savitzky-Golay (11 pts, 3rd order) -> ALS baseline (lambda 1e5, p 0.01) ->

#   SNV within each window (205-1795 and 2705-3095 cm-1)

#   PCA (5 components) on all 240 spectra; LDA on the 5 PC scores.

#   Ellipses: 2 standard deviations of the PC1/PC2 scores of each group.

#   Box plots: median, quartiles, whiskers = 1.5 x IQR; dots = single spectra.

#   p value: Welch's t-test on the 6 site means of each group.

#

# HOW TO RUN (PyCharm):

#   pip install numpy pandas scipy matplotlib openpyxl scikit-learn

#   python Acacia_PCA_LDA_Figure.py          (uses BASE_DIR below)

# OUTPUT: RESULTS\Fig_PCA_LDA.png / .tiff (600 dpi)

#         RESULTS\Fig_MeanSD_CH.png / .tiff (600 dpi) + MeanSD_CH_bands.csv

#

# Band positions in the CH figure (measured on the mean spectrum of each group):

#   'max' = maximum of the band (parabola through the 3 highest points)

#   'sh'  = shoulder: minimum of the 2nd derivative (Savitzky-Golay, 11 pts, 3rd

#           order), the standard way to locate a band without its own maximum

# A light grey shadow marks a band whose EV/NV position differs consistently:

# the positions in the 6 site-mean spectra of EVs and NVs do not overlap.

# =============================================================================

import sys

import csv

from pathlib import Path

import numpy as np

import pandas as pd

import matplotlib

matplotlib.use('Agg')

import matplotlib.pyplot as plt

from matplotlib.patches import Ellipse

from scipy import sparse

from scipy.sparse.linalg import spsolve

from scipy.signal import savgol_filter, find_peaks

from scipy.stats import ttest_ind

from sklearn.decomposition import PCA

from sklearn.discriminant_analysis import LinearDiscriminantAnalysis

BASE_DIR = Path(sys.argv[1]) if len(sys.argv) > 1 else Path(r'C:\Users\Hira Aman\Downloads\CODE 005')

EV_FILE = BASE_DIR / 'EV Acacia' / 'EV_Acacia_Raman_Data.xlsx'

NV_FILE = BASE_DIR / 'Nano vesicles Acacia' / 'NV_Acacia_Raman_Data.xlsx'

OUTPUT_DIR = BASE_DIR / 'RESULTS'

OUTPUT_DIR.mkdir(exist_ok=True)

SITES = ['m1', 'm2', 'm3', 'm4', 'm5', 'm6']

C_EV, C_NV = '#0B5A7E', '#B5541C'

COLS = {0: C_EV, 1: C_NV}

GROUPS = {0: 'EVs', 1: 'NVs'}

N_COMP = 5

# pre-processing window (raw range read, 2 cm-1 grid) and analysed region

WINDOWS = {'fingerprint': {'raw': (200, 1800), 'grid': (205, 1795), 'region': (250, 1790)},

'ch': {'raw': (2700, 3100), 'grid': (2705, 3095), 'region': (2760, 3095)}}

# PC1 is oriented so that its loading is positive at this band (as in the text)

PC1_REF = {'fingerprint': 1580, 'ch': 2960}

TITLES = {'fingerprint': 'Fingerprint (250-1790 cm$^{-1}$)',

'ch': 'CH-stretching (2760-3095 cm$^{-1}$)'}

plt.rcParams.update({'font.family': 'sans-serif',

'font.sans-serif': ['Arial', 'Liberation Sans', 'DejaVu Sans'],

'font.size': 9, 'axes.spines.top': False, 'axes.spines.right': False})

# ----------------------------------------------------------------------------

# PRE-PROCESSING (identical to Acacia_EV_NV_Analysis.py)

# ----------------------------------------------------------------------------

def despike(y, threshold=7.0, m=3):

d = np.diff(y)

med = np.median(d)

mad = np.median(np.abs(d - med))

if mad == 0:

return y.copy()

spikes = np.zeros(len(y), bool)

spikes[1:] = np.abs(0.6745 * (d - med) / mad) > threshold

out = y.copy()

for i in np.where(spikes)[0]:

idx = [j for j in range(max(0, i - m), min(len(y), i + m + 1)) if not spikes[j]]

if idx:

out[i] = y[idx].mean()

return out

def als_baseline(y, lam=1e5, p=0.01, niter=10):

n = len(y)

D = sparse.diags([1.0, -2.0, 1.0], [0, -1, -2], shape=(n, n - 2))

DD = lam * D.dot(D.transpose())

w = np.ones(n)

for _ in range(niter):

z = spsolve((sparse.spdiags(w, 0, n, n) + DD).tocsc(), w * y)

w = p * (y > z) + (1 - p) * (y < z)

return z

def preprocess(x, Y, win):

lo, hi = WINDOWS[win]['raw']

grid = np.arange(WINDOWS[win]['grid'][0], WINDOWS[win]['grid'][1] + 1, 2.0)

m = (x >= lo - 5) & (x <= hi + 5)

corr = []

for y in Y:

ys = savgol_filter(np.interp(grid, x[m], despike(y[m])), 11, 3)

corr.append(ys - als_baseline(ys))

corr = np.array(corr)

return grid, (corr - corr.mean(1, keepdims=True)) / corr.std(1, keepdims=True)

data = {w: [] for w in WINDOWS}

y, sites = [], []

for cls, path in enumerate([EV_FILE, NV_FILE]):

for site in SITES:

df = pd.read_excel(path, sheet_name=site, header=2)

x, Y = df.iloc[:, 0].to_numpy(float), df.iloc[:, 1:].to_numpy(float).T

for w in WINDOWS:

grid, snv = preprocess(x, Y, w)

r0, r1 = WINDOWS[w]['region']

keep = (grid >= r0) & (grid <= r1)

data[w].append(snv[:, keep])

WINDOWS[w]['g'] = grid[keep]

y += [cls] * len(Y)

sites += [f'{GROUPS[cls]}-{site}'] * len(Y)

data = {w: np.vstack(v) for w, v in data.items()}

y, sites = np.array(y), np.array(sites)

print(f'Spectra: {len(y)} (EV {np.sum(y == 0)}, NV {np.sum(y == 1)}), sites: {len(set(sites))}')

def site_means(v):

return {c: np.array([v[sites == s].mean() for s in sorted(set(sites[y == c]))]) for c in (0, 1)}

def stars(p):

return '***' if p < 0.001 else '**' if p < 0.01 else '*' if p < 0.05 else 'n.s.'

# ----------------------------------------------------------------------------

# FIGURE

# ----------------------------------------------------------------------------

fig, axs = plt.subplots(2, 2, figsize=(5.0, 5.3),

gridspec_kw={'width_ratios': [1.1, 0.7], 'wspace': 0.42, 'hspace': 0.45})

letters = iter('abcd')

for row, w in enumerate(['fingerprint', 'ch']):

X, g = data[w], WINDOWS[w]['g']

pca = PCA(n_components=N_COMP).fit(X)

T = pca.transform(X)

T[:, 0] *= np.sign(pca.components_[0][np.argmin(np.abs(g - PC1_REF[w]))])

ev1, ev2 = pca.explained_variance_ratio_[:2] * 100

# --- PCA scores with 2-SD ellipses

ax = axs[row, 0]

for c in (0, 1):

P = T[y == c, :2]

ax.scatter(P[:, 0], P[:, 1], s=6, color=COLS[c], lw=0, alpha=0.85, label=GROUPS[c])

vals, vecs = np.linalg.eigh(np.cov(P.T))

ang = np.degrees(np.arctan2(vecs[1, 1], vecs[0, 1]))

ax.add_patch(Ellipse(P.mean(0), 4 * np.sqrt(vals[1]), 4 * np.sqrt(vals[0]), angle=ang,

fc='none', ec=COLS[c], lw=1.0))

ax.axhline(0, color='k', lw=0.5)

ax.axvline(0, color='k', lw=0.5)

ax.set_xlabel(f'PC1 ({ev1:.1f}%)')

ax.set_ylabel(f'PC2 ({ev2:.1f}%)')

ylo, yhi = ax.get_ylim()

ax.set_ylim(ylo, yhi + 0.28 * (yhi - ylo))

ax.legend(frameon=False, fontsize=7.5, loc='upper left', ncol=2, handletextpad=0.1,

columnspacing=0.8, markerscale=1.5)

ax.set_title(TITLES[w], fontsize=8.5, loc='left')

# --- canonical variable 1 (LDA on the 5 PC scores)

ld = LinearDiscriminantAnalysis().fit(T, y).transform(T).ravel()

ld *= np.sign(ld[y == 1].mean())

ax = axs[row, 1]

rng = np.random.default_rng(0)          # horizontal jitter of the dots only

for c in (0, 1):

v = ld[y == c]

ax.boxplot(v, positions=[c], widths=0.5, showfliers=False, patch_artist=True,

boxprops=dict(fc=COLS[c], alpha=0.30, ec=COLS[c]), medianprops=dict(color=COLS[c]),

whiskerprops=dict(color=COLS[c]), capprops=dict(color=COLS[c]))

ax.scatter(c + rng.uniform(-0.15, 0.15, len(v)), v, s=3, color=COLS[c], lw=0, alpha=0.8)

sm = site_means(ld)

p = ttest_ind(sm[0], sm[1], equal_var=False).pvalue

top = ld.max() + 0.08 * np.ptp(ld)

ax.plot([0, 1], [top, top], color='k', lw=0.6)

ax.text(0.5, top, stars(p), ha='center', va='bottom', fontsize=8)

ax.set_xticks([0, 1])

ax.set_xticklabels(['EVs', 'NVs'])

ax.set_ylabel('Canonical variable 1')

ax.set_ylim(ld.min() - 0.18 * np.ptp(ld), top + 0.15 * np.ptp(ld))

ax.text(0.5, 0.02, "Welch's t-test on site means\n(n = 6), " + stars(p) + f' p = {p:.1e}',

transform=ax.transAxes, ha='center', va='bottom', fontsize=6)

print(f'{w}: PC1 {ev1:.1f} %, PC2 {ev2:.1f} %, LD1 Welch p (site means) = {p:.2e}')

for ax in axs[row]:

ax.text(-0.30 if ax is axs[row, 1] else -0.22, 1.10, f'({next(letters)})',

transform=ax.transAxes, fontsize=10, va='bottom')

fig.subplots_adjust(left=0.13, right=0.98, bottom=0.09, top=0.93)

fig.savefig(OUTPUT_DIR / 'Fig_PCA_LDA.png', dpi=600)

fig.savefig(OUTPUT_DIR / 'Fig_PCA_LDA.tiff', dpi=600, pil_kwargs={'compression': 'tiff_lzw'})

print('Saved in:', OUTPUT_DIR)

# ============================================================================

# CH-STRETCHING REGION: mean +/- SD spectra with band positions

# ============================================================================

CH_OFFSET = 5.5                            # SNV units added to the NV spectrum (display only)

# (nominal band, search half-width, 'max' or 'sh', class, assignment)

CH_BANDS = [

(2853, 10, 'sh', 'Lipids', 'CH2 symmetric stretching'),

(2880, 12, 'sh', 'Lipids', 'CH2 asymmetric stretching'),

(2935, 12, 'max', 'Proteins', 'CH3 symmetric stretching'),

(2960, 10, 'sh', 'Proteins/Lipids', 'CH3 asymmetric stretching'),

(3065, 12, 'max', 'Proteins', 'Aromatic =C-H stretching'),

]

CLASS_SEGMENTS = {'Proteins': [('Proteins', '#7030A0')],

'Lipids': [('Lipids', '#2E75B6')],

'Proteins/Lipids': [('Proteins/', '#7030A0'), ('Lipids', '#2E75B6')]}

def band_position(gr, s, nominal, half, kind):

"""'max': most prominent local maximum within +/-half of the nominal band;

'sh': most prominent local minimum of the 2nd derivative there.

Refined by a parabola through 3 points. None = no such band (not resolved)."""

if kind == 'sh':

s = -savgol_filter(s, 11, 3, deriv=2)

m = np.flatnonzero(np.abs(gr - nominal) <= half)

pk, pr = find_peaks(s[m], prominence=0)

if not len(pk):

return None

i = m[pk[np.argmax(pr['prominences'])]]

a, b, c = s[i - 1], s[i], s[i + 1]

return gr[i] + 0.5 * (a - c) / (a - 2 * b + c) * (gr[1] - gr[0])

def segment_label(ax, xpos, ybase, segments):

"""Vertical label made of coloured text segments stacked upwards."""

yy = ybase

gap = 0.004 * (ax.get_ylim()[1] - ax.get_ylim()[0])

for txt, col in segments:

t = ax.text(xpos, yy, txt, rotation=90, va='bottom', ha='center', fontsize=10, color=col)

ax.figure.canvas.draw()

yy = t.get_window_extent().transformed(ax.transData.inverted()).y1 - gap

plt.rcParams.update({'font.size': 10, 'axes.spines.top': True, 'axes.spines.right': True})

g, X = WINDOWS['ch']['g'], data['ch']

fig, ax = plt.subplots(figsize=(6.5, 6.0))

rows, labels, lines = [], [], []

for nominal, half, kind, cls, assign in CH_BANDS:      # consistent EV/NV differences

pos = {c: band_position(g, X[y == c].mean(0), nominal, half, kind) for c in (0, 1)}

site_pos = {c: [band_position(g, X[sites == k].mean(0), nominal, half, kind)

for k in sorted(set(sites[y == c]))] for c in (0, 1)}

resolved = None not in list(pos.values()) + site_pos[0] + site_pos[1]

consistent = resolved and ((max(site_pos[0]) < min(site_pos[1])) or

(max(site_pos[1]) < min(site_pos[0])))

if consistent:

ax.axvspan(min(pos.values()) - 4, max(pos.values()) + 4, color='0.92', lw=0, zorder=0)

for c in (0, 1):

sp = [v for v in site_pos[c] if v is not None]

rows.append([GROUPS[c], nominal, 'maximum' if kind == 'max' else 'shoulder (2nd derivative)',

'not resolved' if pos[c] is None else round(pos[c], 1),

f'{min(sp):.1f}-{max(sp):.1f} ({len(sp)} of 6 sites)' if sp else 'not resolved',

round(pos[1] - pos[0], 1) if resolved else '', consistent, cls, assign])

for c, off, name in [(0, 0.0, 'EV Acacia on Ag/SiNWs'), (1, CH_OFFSET, 'Nanovesicles Acacia on Ag/SiNWs')]:

mu, sd = X[y == c].mean(0), X[y == c].std(0, ddof=1)

ax.fill_between(g, mu - sd + off, mu + sd + off, color=COLS[c], alpha=0.22, lw=0)

ax.plot(g, mu + off, color=COLS[c], lw=1.0, label=f'{name} (mean \u00b1 SD, n = {np.sum(y == c)})')

for r in rows:

if r[0] != GROUPS[c] or r[3] == 'not resolved':     # unresolved bands: no label

continue

pos = r[3]

top = (mu + sd)[np.abs(g - pos) <= 6].max() + off

txt = f'{pos:.0f}' + (' (sh)' if r[2] != 'maximum' else '')

lines.append((pos, np.interp(pos, g, mu) + off, top + 0.08))

labels.append((pos, top + 0.12, [(txt + '- ', 'k')] + CLASS_SEGMENTS[r[7]]))

for xl, yb, yt in lines:                    # dotted line from the mean spectrum to its label

ax.plot([xl, xl], [yb, yt], ls=(0, (1, 2)), lw=0.7, color='0.35')

ax.set_xlim(2760, 3095)

ax.set_ylim((X[y == 0].mean(0) - X[y == 0].std(0, ddof=1)).min() - 0.3,

(X[y == 1].mean(0) + X[y == 1].std(0, ddof=1)).max() + CH_OFFSET + 6.2)

for lab in labels:                          # after the axis limits are fixed

segment_label(ax, *lab)

ax.tick_params(axis='y', direction='in', length=5, labelleft=False)

ax.tick_params(axis='x', direction='in', length=5)

ax.set_xlabel('Raman Shift (cm$^{-1}$)', fontsize=13)

ax.set_ylabel('Intensity (a.u.)', fontsize=13)

ax.legend(frameon=True, facecolor='white', edgecolor='none', framealpha=1, fontsize=9,

loc='upper left', handlelength=1.5)

fig.tight_layout()

fig.savefig(OUTPUT_DIR / 'Fig_MeanSD_CH.png', dpi=600)

fig.savefig(OUTPUT_DIR / 'Fig_MeanSD_CH.tiff', dpi=600, pil_kwargs={'compression': 'tiff_lzw'})

with open(OUTPUT_DIR / 'MeanSD_CH_bands.csv', 'w', newline='') as fh:

wr = csv.writer(fh)

wr.writerow(['Group', 'Nominal (cm-1)', 'Method', 'Position (cm-1)', '6 sites (min-max)',

'Shift NV-EV (cm-1)', 'Consistent (shadowed)', 'Class', 'Assignment'])

wr.writerows(rows)

for r in rows:

print(r)