# Clean SERS Spectrum Plot

DNA + HaCaT cells  ·  AgSiNW substrate  ·  532 nm, 2.5% laser power, 0.5 s × 4 acc.

# 1. What This Script Does

This script automatically scans all 100 SERS map spectra from files m1–m4 (25 spectra each), scores every spectrum for signal quality, and selects the single cleanest DNA+HaCaT signal. It then plots the full spectrum from 200 to 3200 cm⁻¹ with every detected peak labeled by wavenumber.

The scoring algorithm uses three criteria:

SNR (signal-to-noise ratio): ratio of the baseline-corrected peak max to noise in the silent 2050–2550 cm⁻¹ region.

DNA+HaCaT band matching: how many of 19 known biological Raman bands are detected within ±18 cm⁻¹.

Artifact penalty: spectra with many peaks in the biologically silent 1800–2600 cm⁻¹ region are penalised.

Expected result: m3 spectrum 20 is selected (SNR ≈ 87, 15/19 DNA bands matched, 27 peaks detected). Figure saved as:  clean_sers_spectrum.png

# 2. Requirements and Installation

Python 3.8 or later. Install the three libraries if you haven't already:

pip install numpy scipy matplotlib

In PyCharm: go to File → Settings → Project → Python Interpreter → click + and search each package name.

# 3. How to Run in PyCharm

| Step 1 | Open plot_clean_sers.py in PyCharm. |
| --- | --- |
| Step 2 | Find the line: FOLDER = r"C:\Users\Hira Aman\Desktop\RAMAN MAP 1" |
| Step 3 | Change the path to wherever your 4 map .txt files are stored. |
| Step 4 | Make sure all 4 file names in MAP_FILES exactly match your actual file names. |
| Step 5 | Press Shift+F10 (Run). Progress is printed in the PyCharm console. |
| Step 6 | The figure clean_sers_spectrum.png is saved into FOLDER. |

# 4. What the Figure Shows

| Element | Description |
| --- | --- |
| Spectrum | ALS baseline-corrected, min-max normalised. Full range 200–3200 cm⁻¹. |
| Shaded regions | Colour-coded background: DNA backbone (400–900), fingerprint (900–1300), nucleobases (1300–1800), silent region (1800–2800), CH stretch (2800–3200). |
| Peak labels | Every detected peak labeled with its exact wavenumber in cm⁻¹. Staggered heights prevent overlap. |
| Peak markers | Filled circles at each peak summit connected by a thin line to the label. |
| Title | Shows map name, spectrum number, substrate, and correct laser conditions (2.5%, 0.5 s, 4 acc.). |

# 5. All 27 Detected Peaks (m3 spectrum 20)

These are the peaks your script will label in the figure. Positions are from real data.

| # | Position (cm⁻¹) | Norm. intensity | Assignment |
| --- | --- | --- | --- |
| 1 | 242.4 | 0.282 | — |
| 2 | 405.0 | 0.092 | Ring deformation / sugar |
| 3 | 524.5 | 0.241 | Backbone C-C stretch |
| 4 | 595.7 | 0.155 | G/C ring deformation |
| 5 | 637.2 | 0.138 | G/T ring breathing |
| 6 | 669.5 | 0.169 | G/T ring breathing |
| 7 | 691.8 | 0.199 | Guanine ring breathing |
| 8 | 736.3 | 0.145 | Adenine ring breathing |
| 9 | 772.0 | 0.232 | Cytosine + O-P-O backbone |
| 10 | 913.5 | 0.567 | Backbone C-C stretch |
| 11 | 998.3 | 0.360 | Phenylalanine / ring |
| 12 | 1118.6 | 1.000 | PO₄ / C-N ← DOMINANT SERS PEAK |
| 13 | 1170.7 | 0.660 | C-N (cytosine/adenine) |
| 14 | 1235.3 | 0.092 | Amide III / thymine |
| 15 | 1315.4 | 0.986 | A/T/G in-plane ← 2nd strongest |
| 16 | 1367.9 | 0.511 | T/G C-H deformation |
| 17 | 1402.7 | 0.150 | A/G ring stretch |
| 18 | 1424.7 | 0.198 | A/G ring stretch |
| 19 | 1484.3 | 0.160 | A/C C=N stretch |
| 20 | 1508.6 | 0.171 | A/C C=N stretch |
| 21 | 1544.4 | 0.066 | G/A ring C=C / C=N |
| 22 | 1564.6 | 0.090 | G/A ring C=C / C=N |
| 23 | 1624.8 | 0.126 | Amide I / C=C stretch |
| 24 | 1667.7 | 0.093 | Amide I / C=C stretch |
| 25 | 2148.4 | 0.056 | — (combination band or artefact) |
| 26 | 2930.7 | 0.072 | CH₃ stretch — HaCaT cell lipids |
| 27 | 2977.1 | 0.055 | CH₂ stretch — HaCaT cell lipids |

# 6. Complete Python Code  —  plot_clean_sers.py

Copy everything below into a new file named plot_clean_sers.py in PyCharm, then edit the FOLDER path on line 1 of the file paths section.

"""

plot_clean_sers.py

==================

Loads SERS maps m1–m4, automatically finds the cleanest DNA+HaCaT spectrum

from all 100 spectra, and plots it (200–3200 cm-1) with every peak labeled.

HOW TO RUN IN PyCharm:

1. Edit FOLDER below (the folder that contains all 4 map .txt files).

2. Run (Shift+F10).

3. Figure saved to the same folder as:  clean_sers_spectrum.png

REQUIREMENTS:

pip install numpy scipy matplotlib

"""

import os

import numpy as np

import matplotlib

matplotlib.use("Agg")          # remove this line if you want an interactive window

import matplotlib.pyplot as plt

from scipy.sparse import diags

from scipy.sparse.linalg import spsolve

from scipy.signal import find_peaks

# ══════════════════════════════════════════════════════════════════

#  ▼  EDIT THIS PATH ONLY  ▼

FOLDER = r"C:\Users\Hira Aman\Desktop\RAMAN MAP 1"

#  ▲  EDIT THIS PATH ONLY  ▲

# ══════════════════════════════════════════════════════════════════

FILE_M1 = os.path.join(FOLDER, "m1-DNA-HACAT1-20ng-AgSiNW_532nm_600gr_BC200_50XLF_05s_4a_2-5%_5ul_drop-center.txt")

FILE_M2 = os.path.join(FOLDER, "m2-DNA-HACAT1-20ng-AgSiNW_532nm_600gr_BC200_50XLF_05s_4a_2-5%_5ul_drop-edge.txt")

FILE_M3 = os.path.join(FOLDER, "m3-DNA-HACAT1-20ng-AgSiNW_532nm_600gr_BC200_50XLF_05s_4a_2-5%_5ul_drop-center.txt")

FILE_M4 = os.path.join(FOLDER, "m4-DNA-HACAT1-20ng-AgSiNW_532nm_600gr_BC200_50XLF_05s_4a_2-5%_5ul_drop-center.txt")

MAP_FILES = {"m1": FILE_M1, "m2": FILE_M2, "m3": FILE_M3, "m4": FILE_M4}

# ── Known DNA + HaCaT Raman band positions (literature) ──────────

KNOWN_BANDS = [650, 720, 752, 782, 828, 932, 1001, 1092,

1178, 1240, 1310, 1375, 1425, 1484, 1575, 1640,

2870, 2930, 2960]

KNOWN_TOL   = 18    # cm-1 match window for scoring

MATCH_TOL   = 18    # cm-1 window used for peak finding

# ── Spectral region colours and labels (background shading) ──────

REGIONS = {

(200,  400):  ("#EBF5FB", "200–400"),

(400,  900):  ("#FEF9E7", "DNA backbone\n400–900"),

(900,  1300): ("#EAFAF1", "Fingerprint\n900–1300"),

(1300, 1800): ("#FDF2F8", "Nucleobases\n1300–1800"),

(1800, 2800): ("#F8F9FA", "Silent region\n1800–2800"),

(2800, 3200): ("#EBF5FB", "CH stretch\n2800–3200"),

}

# ── Assignment table ──────────────────────────────────────────────

ASSIGN = {

(400,  460):  "Ring deformation / sugar",

(460,  510):  "Sugar-phosphate backbone",

(510,  560):  "Backbone C-C stretch",

(560,  615):  "G/C ring deformation",

(615,  680):  "G/T ring breathing",

(680,  710):  "Guanine ring breathing",

(710,  750):  "Adenine ring breathing",

(750,  800):  "Cytosine + O-P-O backbone",

(800,  860):  "Backbone O-P-O stretch",

(860,  920):  "Backbone C-C stretch",

(920,  970):  "Sugar C-O stretch",

(970,  1020): "Phenylalanine / ring",

(1020, 1075): "C-N stretch / ring",

(1075, 1115): "PO₄ symmetric stretch",

(1115, 1145): "PO₄ / C-N",

(1145, 1200): "C-N (cytosine/adenine)",

(1200, 1270): "Amide III / thymine",

(1270, 1340): "A/T/G in-plane",

(1340, 1400): "T/G C-H deformation",

(1400, 1455): "A/G ring stretch",

(1455, 1510): "A/C C=N stretch",

(1510, 1595): "G/A ring C=C / C=N",

(1595, 1680): "Amide I / C=C stretch",

(1680, 1800): "C=O stretch (nucleobase)",

(2800, 2900): "CH₂ symmetric stretch (HaCaT)",

(2900, 2970): "CH₃ symmetric stretch (HaCaT)",

(2970, 3100): "CH₂ asymmetric stretch (HaCaT)",

}

def get_assignment(wn_val):

for (lo, hi), label in ASSIGN.items():

if lo <= wn_val < hi:

return label

return "—"

# ═════════════════════════════════════════════════════════════════

#  PROCESSING FUNCTIONS

# ═════════════════════════════════════════════════════════════════

def load_map(filepath):

"""Load a LabSpec6 map .txt file. Returns (wavenumber_array, spectra_2D)."""

rows = []

with open(filepath, "r", encoding="utf-8", errors="replace") as fh:

for line in fh:

line = line.strip()

if not line or line.startswith("#"):

continue

parts = line.replace(",", ".").split("\t")

if len(parts) < 3:

parts = line.replace(",", ".").split()

try:

rows.append([float(v) for v in parts])

except ValueError:

continue

wavenumbers = np.array(rows[0])

n_pts = len(wavenumbers)

spectra = []

for row in rows[1:]:

arr = np.array(row)

if   len(arr) == n_pts + 2:  spectra.append(arr[2:])

elif len(arr) == n_pts:       spectra.append(arr)

else:                         spectra.append(arr[-n_pts:])

return wavenumbers, np.array(spectra)

def als_baseline(y, lam=1e5, p=0.005, n_iter=15):

"""Asymmetric Least Squares baseline correction."""

L = len(y)

D = diags([1, -2, 1], [0, 1, 2], shape=(L - 2, L), dtype=float)

w = np.ones(L)

for _ in range(n_iter):

W = diags(w, 0, shape=(L, L), dtype=float)

z = spsolve((W + lam * D.T.dot(D)).tocsr(), w * y)

w = p * (y > z) + (1 - p) * (y <= z)

return z

def process_spectrum(wn, sp_raw):

"""ALS baseline correction + min-max normalisation."""

z  = als_baseline(sp_raw)

bc = np.clip(sp_raw - z, 0, None)

sp_max = bc.max()

if sp_max <= 0:

return bc, 0.0, 0.0

sp_n = bc / sp_max

return sp_n, sp_max, z.max()

def find_spectrum_peaks(wn, sp_n, height=0.05, prominence=0.04):

"""Detect genuine Raman peaks (above noise floor)."""

step = float(np.diff(wn).mean())

min_dist = max(5, int(15 / step))

idxs, props = find_peaks(sp_n, height=height, prominence=prominence,

distance=min_dist)

return wn[idxs], sp_n[idxs]

def score_spectrum(wn, sp_raw):

"""Score a spectrum for 'cleanliness': high SNR + many known DNA bands."""

sp_n, sp_max, _ = process_spectrum(wn, sp_raw)

if sp_max <= 0:

return -1, sp_n, sp_max

# SNR: signal max vs noise in the 'silent' 2050–2550 cm-1 region

noise_mask = (wn > 2050) & (wn < 2550)

noise_std  = (sp_raw - als_baseline(sp_raw))[noise_mask].std()

if noise_std < 1:

noise_std = 1

snr = sp_max / noise_std

# Count how many known DNA+HaCaT bands are present

pk_wns, _ = find_spectrum_peaks(wn, sp_n)

n_matched  = sum(1 for k in KNOWN_BANDS

if pk_wns.size > 0 and np.abs(pk_wns - k).min() < KNOWN_TOL)

# Penalise artifact peaks in 1800–2600 cm-1 (biologically silent region)

artifact_mask = (pk_wns > 1800) & (pk_wns < 2600)

n_artifacts   = int(artifact_mask.sum())

score = n_matched * snr / (1 + n_artifacts * 2)

return score, sp_n, sp_max

# ═════════════════════════════════════════════════════════════════

#  MAIN

# ═════════════════════════════════════════════════════════════════

def main():

matplotlib.rcParams.update({"font.family": "DejaVu Sans",

"figure.facecolor": "white"})

print("=" * 60)

print("Scanning all 100 SERS spectra for the cleanest signal ...")

print("=" * 60)

best = {"score": -1}

for map_name, filepath in MAP_FILES.items():

print(f"  Loading {map_name} ...")

wn, spectra = load_map(filepath)

for i, sp_raw in enumerate(spectra):

score, sp_n, sp_max = score_spectrum(wn, sp_raw)

if score > best["score"]:

pk_wns, pk_ints = find_spectrum_peaks(wn, sp_n)

best = {

"score":   score,

"map":     map_name,

"sp_idx":  i + 1,

"wn":      wn,

"sp_n":    sp_n,

"sp_max":  sp_max,

"pk_wns":  pk_wns,

"pk_ints": pk_ints,

}

print(f"\n  Best spectrum : {best['map']} · spectrum {best['sp_idx']}")

print(f"  Raw max count : {best['sp_max']:.0f}")

n_bio = sum(1 for k in KNOWN_BANDS

if best["pk_wns"].size > 0 and

np.abs(best["pk_wns"] - k).min() < KNOWN_TOL)

print(f"  Peaks matched : {n_bio}/19 known DNA+HaCaT bands")

print(f"  Peaks detected: {len(best['pk_wns'])} total")

# ── Print peak table ─────────────────────────────────────────

print("\n  #   Position (cm-1)  Norm. intensity  Assignment")

print("  " + "-" * 62)

for j, (w, h) in enumerate(zip(best["pk_wns"], best["pk_ints"]), 1):

print(f"  {j:<4}{w:<20.1f}{h:<17.4f}{get_assignment(w)}")

# ═══════════════════════════════════════════════════════════════

#  FIGURE

# ═══════════════════════════════════════════════════════════════

wn   = best["wn"]

sp_n = best["sp_n"]

fig, ax = plt.subplots(figsize=(18, 7))

fig.patch.set_facecolor("white")

ax.set_facecolor("white")

# Shaded spectral regions

for (lo, hi), (color, label) in REGIONS.items():

ax.axvspan(lo, hi, color=color, alpha=0.55, lw=0)

mid = (lo + hi) / 2

if hi - lo > 150:

ax.text(mid, 1.28, label, fontsize=7.5, ha="center", va="bottom",

color="grey", style="italic")

# Spectrum fill + line

mask = (wn >= 200) & (wn <= 3200)

ax.fill_between(wn[mask], sp_n[mask], alpha=0.15, color="#1A5276")

ax.plot(wn[mask], sp_n[mask], color="#1A5276", lw=1.5, zorder=5)

# Peak markers + staggered labels

pks = [(w, h) for w, h in zip(best["pk_wns"], best["pk_ints"])

if 200 < w < 3200]

pks.sort(key=lambda x: x[0])

label_y = [1.10, 1.18, 1.10, 1.18]

prev_x = -9999

level_idx = 0

for w, h in pks:

gap = w - prev_x

level_idx = (level_idx + 1) % 4 if gap < 45 else 0

y_lbl = label_y[level_idx]

ax.plot(w, h, "o", color="#1A5276", ms=5.5, zorder=8,

markeredgecolor="white", markeredgewidth=0.6)

ax.plot([w, w], [h + 0.02, y_lbl - 0.04],

color="#1A5276", lw=0.7, alpha=0.55)

rot = 90 if gap < 60 else 0

ax.text(w, y_lbl, f"{w:.0f}", fontsize=7.5, ha="center", va="bottom",

color="#1A5276", fontweight="bold", rotation=rot)

ax.axvline(w, color="#1A5276", lw=0.45, ls="--", alpha=0.25, zorder=3)

prev_x = w

ax.set_xlim(200, 3200)

ax.set_ylim(-0.05, 1.50)

ax.set_xlabel("Raman shift (cm⁻¹)", fontsize=13, fontweight="bold")

ax.set_ylabel("Normalised intensity (a.u.)", fontsize=13, fontweight="bold")

ax.set_title(

f"DNA + HaCaT cells  |  SERS  |  {best['map']} spectrum {best['sp_idx']}"

f"  |  AgSiNW substrate  |  532 nm · 2.5% laser power · 0.5 s × 4 acc.\n"

f"Cleanest signal from 100 spectra (m1–m4)  ·  "

f"ALS baseline corrected  ·  {len(pks)} peaks detected",

fontsize=11, fontweight="bold"

)

ax.spines["top"].set_visible(False)

ax.spines["right"].set_visible(False)

ax.tick_params(labelsize=10)

ax.grid(True, alpha=0.10, lw=0.5)

out_path = os.path.join(FOLDER, "clean_sers_spectrum.png")

fig.savefig(out_path, dpi=180, bbox_inches="tight", facecolor="white")

plt.close(fig)

print(f"\n  Figure saved: {out_path}")

print("\nDone.")

if __name__ == "__main__":

main()