# Acacia EVs vs NVs - CaF2 vs Ag/SiNWs raw spectra (EV or NV)

How to use:

1. In PyCharm: File > New > Python File, name it EV_Acacia_TwoSpectra.py.

2. Select all the code below (from the first "# ===" line to the last line), copy and paste it into that file.

3. In the PyCharm Terminal, install the packages once:  pip install numpy scipy matplotlib

4. Run it twice in the Terminal, from the folder with the .txt files:

EV:  python EV_Acacia_TwoSpectra.py "S2-EV_Acacia-dropCaF2_532nm_1800gr_10_50xLF_BC200_1s_4a.txt" "S2-EV_ACACIA-AgSiNW_532nm_1800gr_BC200_50XLF_1s_4a_10%.txt"

NV:  python EV_Acacia_TwoSpectra.py "s1-EV_nano_Acacia-dropCaF2_532nm_1800gr_10_50xLF_BC200_1s_4a.txt" "s4a-EV_nano_ACACIA-20ng-AgSiNW_532nm_1800gr_BC200_50XLF_1s_4a_10%.txt"

Outputs: CaF2_AgSiNW_EV.png/.tiff/_peaks.csv and CaF2_AgSiNW_NV.png/.tiff/_peaks.csv

------------------------------------------------------------

# =============================================================================

# ACACIA EVs or NVs  |  CaF2 vs Ag/SiNWs  |  two raw spectra with peak assignments

#

# Both measured spectra on one axis (raw counts: no smoothing, no baseline, no

# normalisation), broken x-axis over the 1800-2700 cm-1 gap, dotted line from

# each labelled peak up to its label (wavenumber + biomolecular class). The

# sample names are written inside the plot, next to their spectra; the y-axis

# has no numbers.

#

# The sample (EV or NV) is read from the Ag/SiNW file name ('nano' = NV).

#   EV: S2-EV_Acacia-dropCaF2 ...txt          + S2-EV_ACACIA-AgSiNW ...txt

#   NV: s1-EV_nano_Acacia-dropCaF2 ...txt     + s4a-EV_nano_ACACIA-20ng-AgSiNW ...txt

#

# Peak position = most prominent local maximum of the Ag/SiNW spectrum within

# +/-12 cm-1 of the nominal band, found on a lightly smoothed copy (Savitzky-Golay,

# 11 pts, 3rd order; used ONLY to locate the maximum). Same method as

# EV_NV_Spectra_Comparison.py, so both figures give the same numbers. The CSV

# also lists each peak's prominence relative to the noise of the spectrum.

#

# Assignment sources:

#   [P]  Roda F., Picciolini S., ... Gualerzi A., Bedoni M. Nanomaterials 2023, 13, 699

#   [M]  Movasaghi Z., Rehman S., Rehman I.U. Appl. Spectrosc. Rev. 2007, 42, 493-541

#   [T]  Talari A.C.S. et al. Appl. Spectrosc. Rev. 2015, 50, 46-111

#   [C]  Czamara K. et al. J. Raman Spectrosc. 2015, 46, 4-20

#   [G]  Gualerzi A. et al. Sci. Rep. 2017, 7, 9820

#

# HOW TO RUN (PyCharm):

#   pip install numpy scipy matplotlib

#   python EV_Acacia_TwoSpectra.py  <CaF2 file .txt>  <Ag/SiNW file .txt>

# OUTPUT: CaF2_AgSiNW_<EV|NV>.png / .tiff (600 dpi) + CaF2_AgSiNW_<EV|NV>_peaks.csv

# =============================================================================

import sys

import csv

from pathlib import Path

import numpy as np

import matplotlib

matplotlib.use('Agg')

import matplotlib.pyplot as plt

from scipy.signal import savgol_filter, find_peaks

# ----------------------------------------------------------------------------

# INPUT / OUTPUT

# ----------------------------------------------------------------------------

FILE_CAF2 = Path(sys.argv[1] if len(sys.argv) > 1 else

'S2-EV_Acacia-dropCaF2_532nm_1800gr_10_50xLF_BC200_1s_4a.txt')

FILE_AG = Path(sys.argv[2] if len(sys.argv) > 2 else

'S2-EV_ACACIA-AgSiNW_532nm_1800gr_BC200_50XLF_1s_4a_10%.txt')

TAG = 'NV' if 'nano' in FILE_AG.name.lower() else 'EV'

if ('nano' in FILE_CAF2.name.lower()) != (TAG == 'NV'):

sys.exit('CaF2 and Ag/SiNW files are from different samples - check the file names')

OUT = f'CaF2_AgSiNW_{TAG}'

# ----------------------------------------------------------------------------

# SETTINGS

# ----------------------------------------------------------------------------

LOW_MIN, LOW_MAX = 400, 1800

HIGH_MIN, HIGH_MAX = 2700, 3100

SEARCH_HALF = 12         # cm-1 window to locate the measured maximum

SAMPLE_NAME = {'EV': 'EV Acacia', 'NV': 'Nanovesicles Acacia'}[TAG]

# counts added to the Ag/SiNW spectrum (display only) so the traces do not overlap

AG_OFFSET = {'EV': 110, 'NV': 0}[TAG]

# (nominal position, class, assignment, source)

PEAK_SETS = {

'EV': [

(519,  'Si substrate',  'Crystalline silicon optical phonon (520.7 cm-1 Si band); from SiNW', '[P]'),

(831,  'Protein/NA',    'Tyrosine Fermi doublet (828/850) / O-P-O asymmetric stretch (828)', '[M][T]'),

(1001, 'Protein',       'Phenylalanine symmetric ring breathing (1002-1004)', '[M][T][G]'),

(1031, 'Protein',       'Phenylalanine C-H in-plane bending (1030-1034)', '[M][T]'),

(1166, 'Protein',       'Tyrosine C-H in-plane bending (1170-1175)', '[M][T]'),

(1355, 'Protein',       'Tryptophan / CH deformation (1340-1380); broad band', '[M][T]'),

(1582, 'Protein',       'Phenylalanine C=C bending (1584)', '[M][T]'),

(1604, 'Protein',       'Phenylalanine / tyrosine ring C=C stretch (1600-1607)', '[M][T][G]'),

(1724, 'Lipid',         'C=O ester stretching of (phospho)lipids (1720-1745)', '[M][T][C]'),

(2929, 'Protein/Lipid', 'CH2/CH3 stretching (2925-2935)', '[P][M][C][G]'),

],

'NV': [

(519,  'Si substrate',  'Crystalline silicon optical phonon (520.7 cm-1 Si band); from SiNW', '[P]'),

(612,  'Protein',       'C-C twisting of phenylalanine (~621); weak Si 2nd-order phonon may overlap', '[M][T]'),

(797,  'Nucleic acid',  'O-P-O backbone / phosphodiester stretching (785-810)', '[M][T]'),

(826,  'Protein/NA',    'Tyrosine Fermi doublet (828/850) / O-P-O asymmetric stretch (828)', '[M][T]'),

(921,  'Protein/Carb.', 'C-C stretch of proline ring / glucose (920)', '[M][T]'),

(1001, 'Protein',       'Phenylalanine symmetric ring breathing (1002-1004)', '[M][T][G]'),

(1031, 'Protein',       'Phenylalanine C-H in-plane bending (1030-1031)', '[M][T]'),

(1240, 'Protein',       'Amide III (1230-1250); asym. PO2- of nucleic acids may overlap', '[M][T][G]'),

(1386, 'Lipid/Protein', 'CH3 symmetric deformation (~1380) / COO- symmetric stretch', '[M][T]'),

(1607, 'Protein',       'Phenylalanine / tyrosine ring C=C stretch (1605-1607)', '[M][T][G]'),

(1624, 'Protein',       'Amide I low-frequency side / Trp-Tyr C=C (1617-1630)', '[M][T]'),

(2936, 'Protein/Lipid', 'CH3 symmetric stretching (2930-2940), mainly proteins', '[P][M][C][G]'),

],

}

PEAKS = PEAK_SETS[TAG]

C_AG = '#E8383D'     # red  : on Ag/SiNWs

C_CAF2 = '#3A3A3A'   # dark grey : on CaF2

CLASS_COL = {        # label colour by class

'Protein': '#1F6FB4', 'Protein/NA': '#6A3D9A', 'Nucleic acid': '#8E44AD',

'Protein/Carb.': '#1F6FB4', 'Lipid/Protein': '#D35400',

'Protein/Lipid': '#D35400', 'Lipid': '#C0392B', 'Si substrate': '#7F7F7F',

}

# ----------------------------------------------------------------------------

# DATA

# ----------------------------------------------------------------------------

caf2 = np.loadtxt(FILE_CAF2)

ag = np.loadtxt(FILE_AG)

ag_s = savgol_filter(ag[:, 1], 11, 3)          # only to locate the maxima

noise = np.std(ag[:, 1] - ag_s)

ag[:, 1] += AG_OFFSET                          # display only

rows = []

for nominal, cls, assign, ref in PEAKS:

k = np.flatnonzero(np.abs(ag[:, 0] - nominal) <= SEARCH_HALF)

pk, pr = find_peaks(ag_s[k], prominence=0)

j = np.argmax(pr['prominences'])

i = k[pk[j]]

rows.append((nominal, round(float(ag[i, 0])), cls, assign, ref,

round(float(pr['prominences'][j] / noise), 1)))

with open(f'{OUT}_peaks.csv', 'w', newline='') as fh:

w = csv.writer(fh)

w.writerow(['Nominal (cm-1)', 'Measured Ag/SiNW (cm-1)', 'Class', 'Assignment', 'Source',

'Prominence / noise SD'])

w.writerows(rows)

# ----------------------------------------------------------------------------

# PLOT

# ----------------------------------------------------------------------------

# font sizes (pt) - large enough to stay readable when the figure is reduced to

# one journal column

FS_PEAK, FS_NAME, FS_AXIS, FS_TICK = 12, 15, 17, 15

plt.rcParams.update({'font.family': 'sans-serif',

'font.sans-serif': ['Arial', 'Liberation Sans', 'DejaVu Sans'],

'font.size': FS_TICK})

fig, (axL, axR) = plt.subplots(

1, 2, sharey=True, figsize=(9, 6.5),

gridspec_kw={'width_ratios': [LOW_MAX - LOW_MIN, HIGH_MAX - HIGH_MIN],

'wspace': 0.03})

def in_view(d):

x = d[:, 0]

return d[((x >= LOW_MIN) & (x <= LOW_MAX)) | ((x >= HIGH_MIN) & (x <= HIGH_MAX)), 1]

y_min = min(in_view(caf2).min(), in_view(ag).min())

y_max = max(in_view(caf2).max(), in_view(ag).max())

y_top = y_max + 0.70 * (y_max - y_min)       # head-room for the rotated labels

y_bot = y_min - 0.12 * (y_max - y_min)       # room for the CaF2 sample name

axL.set_ylim(y_bot, y_top)

for ax, lo, hi in [(axL, LOW_MIN, LOW_MAX), (axR, HIGH_MIN, HIGH_MAX)]:

for d, col in [(caf2, C_CAF2), (ag, C_AG)]:

m = (d[:, 0] >= lo) & (d[:, 0] <= hi)

ax.plot(d[m, 0], d[m, 1], color=col, lw=0.8)

ax.set_xlim(lo, hi)

pos = [r[1] for r in rows]

for k, (nominal, measured, cls, _, _, _) in enumerate(rows):

if not lo <= measured <= hi:

continue

# close neighbour on the left -> put this label right of its line

near_left = k > 0 and measured - pos[k - 1] < 45

ha = 'left' if (near_left or measured - lo < 30) else 'right'

t = ax.text(measured, y_top - 0.01 * (y_top - y_min), f'{measured}  {cls}',

rotation=90, ha=ha, va='top', fontsize=FS_PEAK, color=CLASS_COL[cls])

fig.canvas.draw()

label_bottom = t.get_window_extent().transformed(ax.transData.inverted()).y0

peak_top = ag[np.abs(ag[:, 0] - measured) <= 3, 1].max()

ax.plot([measured, measured], [peak_top + 0.01 * (y_top - y_min), label_bottom],

color='k', ls=':', lw=1.0)

ax.tick_params(direction='in', top=True, length=6, width=1.0, labelsize=FS_TICK)

axL.spines['right'].set_visible(False)

axR.spines['left'].set_visible(False)

axL.tick_params(labelleft=False)                     # no intensity values

axR.tick_params(left=False, right=True, labelleft=False)

axL.set_xticks([400, 600, 800, 1000, 1200, 1400, 1600])

axR.set_xticks([2800, 3000])

# sample names inside the plot: Ag/SiNW name just below its spectrum,

# CaF2 name just below its spectrum (both at the left, from x = 420)

x_lab = (420, 760)

m = (ag[:, 0] >= x_lab[0]) & (ag[:, 0] <= x_lab[1])

axL.text(x_lab[0], ag[m, 1].min() - 0.015 * (y_top - y_min), f'{SAMPLE_NAME} on Ag/SiNWs',

color=C_AG, fontsize=FS_NAME, ha='left', va='top')

m = (caf2[:, 0] >= x_lab[0]) & (caf2[:, 0] <= x_lab[1])

axL.text(x_lab[0], caf2[m, 1].min() - 0.015 * (y_top - y_min), f'{SAMPLE_NAME} on CaF$_2$',

color=C_CAF2, fontsize=FS_NAME, ha='left', va='top')

# // break marks

d = 0.012

kw = dict(color='k', clip_on=False, lw=1.0)

for ax, x0 in [(axL, 1), (axR, 0)]:

dx = d * (HIGH_MAX - HIGH_MIN) / (LOW_MAX - LOW_MIN) if ax is axL else d

for y0 in (0, 1):

ax.plot((x0 - dx, x0 + dx), (y0 - 2 * d, y0 + 2 * d), transform=ax.transAxes, **kw)

axL.set_ylabel('Raman Intensity (a.u.)', fontsize=FS_AXIS)

fig.text(0.53, 0.015, 'Wavenumber (cm$^{-1}$)', ha='center', fontsize=FS_AXIS)

fig.subplots_adjust(left=0.07, right=0.97, bottom=0.12, top=0.97)

fig.savefig(f'{OUT}.png', dpi=600)

fig.savefig(f'{OUT}.tiff', dpi=600, pil_kwargs={'compression': 'tiff_lzw'})

for r in rows:

print(r)

print(f'saved {OUT}.png / .tiff and {OUT}_peaks.csv')