# Acacia EVs vs NVs - single-spectra comparison script

How to use:

1. In PyCharm: File > New > Python File, name it EV_NV_Spectra_Comparison.py.

2. Select all the code below (from the first "# ===" line to the last line), copy and paste it into that file.

3. In the PyCharm Terminal, install the packages once:  pip install numpy scipy matplotlib

4. Put the two .txt files (S1-EV_ACACIA... and s4a-EV_nano_ACACIA...) in the same folder as this script, or give their paths after the script name, and run it. The figure (PNG + TIFF, 600 dpi) and the peak table EV_NV_Comparison_peaks.csv are saved next to the script.

------------------------------------------------------------

# =============================================================================

# ACACIA EVs vs NVs  |  single SERS spectra on Ag/SiNWs  |  comparison figure

#

#   S1-EV_ACACIA-AgSiNW ...txt              -> Acacia EVs  (black)

#   s4a-EV_nano_ACACIA-20ng-AgSiNW ...txt   -> Acacia NVs  (red)

#

# Raw counts are plotted exactly as recorded: no smoothing, no baseline

# correction, no normalisation.

# The NV spectrum is shifted up by a constant (OFFSET) so the two traces do not

# overlap; the y-axis therefore has no numbers, as usual for stacked spectra.

#

# Every peak label is measured from the data: the maximum of the lightly

# smoothed spectrum (Savitzky-Golay 11 pts, 3rd order; used ONLY to locate the

# maximum) after removing a straight local background over +/-30 cm-1.

# Peaks rising > 2.5 x the noise of that spectrum are labelled normally; weaker

# features are labelled in italics with '~' (tentative), as stated on the figure.

# All measured values are written to EV_NV_Comparison_peaks.csv.

#

# HOW TO RUN (PyCharm):

#   pip install numpy scipy matplotlib

#   python EV_NV_Spectra_Comparison.py  <EV file .txt>  <NV file .txt>

# =============================================================================

import sys

import csv

from pathlib import Path

import numpy as np

import matplotlib

matplotlib.use('Agg')

import matplotlib.pyplot as plt

from matplotlib.patches import Rectangle

from scipy.signal import savgol_filter, find_peaks

EV_FILE = Path(sys.argv[1] if len(sys.argv) > 1 else

'S1-EV_ACACIA-AgSiNW_532nm_1800gr_BC200_50XLF_1s_4a_10%.txt')

NV_FILE = Path(sys.argv[2] if len(sys.argv) > 2 else

's4a-EV_nano_ACACIA-20ng-AgSiNW_532nm_1800gr_BC200_50XLF_1s_4a_10%.txt')

OUT = 'EV_NV_Comparison'

X_MIN, X_MAX = 450, 1800

OFFSET = 560            # counts added to the NV spectrum (display only)

SNR_MIN = 2.5           # below this peak height / noise a label is marked '~' (weak)

BOXES = [(775, 890), (1195, 1320), (1515, 1690)]   # dashed comparison regions

# nominal position, class shown on the figure, assignment (for the CSV)

# where = 'up', 'up-left' or 'up-right': label above the peak (shifted aside for close pairs);

# 'pair-1' + 'pair-2': two bands of a doublet drawn as one label (e.g. 825/852)

PEAKS = {

'NV': [

(519, 'Si', 'Si substrate (crystalline silicon phonon)', 'up'),

(797, 'Nucleic acids', 'O-P-O backbone stretching', 'up-left'),

(826, 'Proteins', 'Tyr Fermi doublet', 'pair-1'),

(855, 'Proteins', 'Tyr Fermi doublet', 'pair-2'),

(1001, 'Proteins', 'Phe ring breathing', 'up'),

(1034, 'Proteins', 'Phe C-H in-plane bending', 'up'),

(1241, 'Proteins', 'Amide III', 'up'),

(1387, 'Lipids/NA', 'CH3 deformation (lipids) / nucleic-acid bases; COO- on Ag', 'up'),

(1607, 'Proteins', 'Phe/Tyr ring C=C stretching', 'up-left'),

(1624, 'Proteins', 'Tyr/Trp ring C=C stretching', 'up-right'),

],

'EV': [

(519, 'Si', 'Si substrate (crystalline silicon phonon)', 'up'),

(1001, 'Proteins', 'Phe ring breathing', 'up'),

(1043, 'Proteins', 'Phe C-H in-plane bending', 'up'),

(1206, 'Proteins', 'Phe/Tyr C-C6H5 stretching', 'up'),

(1284, 'Proteins', 'Amide III', 'up'),

(1380, 'Lipids/NA', 'CH3 deformation (lipids) / nucleic-acid bases; COO- on Ag', 'up'),

(1604, 'Proteins', 'Phe/Tyr ring C=C stretching', 'up-left'),

(1628, 'Proteins', 'Tyr/Trp ring C=C stretching (shoulder)', 'up-right'),

],

}

CLASS_COL = {'Proteins': '#7030A0', 'Nucleic acids': '#1F3864', 'Lipids/NA': '#2E75B6', 'Si': 'k'}

C_EV, C_NV = 'k', '#FF0000'

plt.rcParams.update({'font.family': 'sans-serif',

'font.sans-serif': ['Arial', 'Liberation Sans', 'DejaVu Sans'],

'font.size': 11})

def load(path):

d = np.loadtxt(path)

m = (d[:, 0] >= X_MIN) & (d[:, 0] <= X_MAX)

return d[m, 0], d[m, 1]

def measure(x, y, nominal, noise, shoulder=False):

"""Measured peak position, height above local line and SNR.

Position = most prominent local maximum of the smoothed spectrum within

+/-12 cm-1 of the nominal value (works for peaks and shoulders on a slope).

Height = smoothed value at that position minus a straight local background

drawn between the spectrum at -30 and +30 cm-1; for a shoulder (on the flank

of a larger band) the prominence of the local maximum is used instead."""

ys = savgol_filter(y, 11, 3)

k = np.abs(x - nominal) <= 12

pk, pr = find_peaks(ys[k], prominence=0)

if len(pk):

i = np.flatnonzero(k)[pk[np.argmax(pr['prominences'])]]

else:

i = np.flatnonzero(k)[np.argmax(ys[k])]

w = np.abs(x - x[i]) <= 30

xw, yw = x[w], ys[w]

h = ys[i] - np.interp(x[i], [xw[0], xw[-1]], [yw[:6].mean(), yw[-6:].mean()])

if shoulder:      # on the flank of a larger band: use the prominence of the local maximum

h = float(pr['prominences'].max()) if len(pk) else 0.0

return float(x[i]), float(h), float(h / noise)

spectra = {'EV': load(EV_FILE), 'NV': load(NV_FILE)}

noise = {s: np.std(np.diff(y)) / np.sqrt(2) for s, (x, y) in spectra.items()}

def two_colour_label(ax, xpos, ybase, num, cls, ha, weak):

"""Vertical label: wavenumber in black, then '-Class' in the class colour."""

style = 'italic' if weak else 'normal'

t = ax.text(xpos, ybase, num, rotation=90, va='bottom', ha=ha, fontsize=10,

color='k', style=style)

if cls == 'Si':

return

ax.figure.canvas.draw()

bb = t.get_window_extent().transformed(ax.transData.inverted())

gap = 0.004 * (ax.get_ylim()[1] - ax.get_ylim()[0])   # remove the font's end padding

ax.text(xpos, bb.y1 - gap, f'-{cls}', rotation=90, va='bottom', ha=ha, fontsize=10,

color=CLASS_COL[cls], style=style)

rows, labels = [], []

fig, ax = plt.subplots(figsize=(8.5, 6.5))

for s, col, off in [('EV', C_EV, 0), ('NV', C_NV, OFFSET)]:

x, y = spectra[s]

ys_plot = y                        # raw counts, plotted unchanged

ax.plot(x, ys_plot + off, color=col, lw=0.7)

for nominal, cls, assign, where in PEAKS[s]:

sh = '(shoulder)' in assign

pos, h, snr = measure(x, y, nominal, noise[s], shoulder=sh)

weak = snr < SNR_MIN

rows.append([s, nominal, round(pos, 1), round(h, 1), round(snr, 1), cls, assign,

'weak - tentative (~)' if weak else 'yes'])

top = ys_plot[np.abs(x - pos) <= 4].max() + off

txt = f'{"~" if weak else ""}{pos:.0f}{" sh" if sh else ""}'

if where == 'pair-1':              # first band of a doublet: drawn with the second

pair = (pos, txt, top, weak)

continue

if where == 'pair-2':

txt = f'{pair[1]}/{txt}'

pos, top, weak, where = (pair[0] + pos) / 2, max(pair[2], top), pair[3] or weak, 'up'

top = ys_plot[np.abs(x - pos) <= 25].max() + off if where != 'up' else top

dx = {'up': 0, 'up-left': -4, 'up-right': 4}[where]

labels.append((pos + dx, top + 8, txt, cls,

{'up': 'center', 'up-left': 'right', 'up-right': 'left'}[where], weak))

y_lo = spectra['EV'][1].min() - 15

y_hi = spectra['NV'][1].max() + OFFSET + 190

for lo, hi in BOXES:

ax.add_patch(Rectangle((lo, y_lo + 5), hi - lo, y_hi - y_lo - 20, fill=False,

ls='--', lw=0.8, ec='#1F3864'))

ax.set_xlim(X_MIN, X_MAX)

ax.set_ylim(y_lo, y_hi)

for lab in labels:                       # after the axis limits are fixed

two_colour_label(ax, *lab)

ax.set_yticks([])

ax.tick_params(axis='x', direction='in', length=5, top=False)

ax.set_xlabel('Raman Shift (cm$^{-1}$)', fontsize=13)

ax.set_ylabel('Intensity (a.u.)', fontsize=13)

ax.text(0.99, 1.012, 'Nanovesicles Acacia on Ag/SiNWs', transform=ax.transAxes,

ha='right', va='bottom', color=C_NV, fontsize=12)

ax.text(0.99, 0.015, 'EV Acacia on Ag/SiNWs', transform=ax.transAxes,

ha='right', va='bottom', color=C_EV, fontsize=12)

ax.text(0.01, 0.60, f'~ weak band (< {SNR_MIN} x noise), tentative;  sh = shoulder',

transform=ax.transAxes, ha='left', va='top', fontsize=9, style='italic',

bbox=dict(fc='white', ec='none', pad=1), zorder=5)

fig.tight_layout()

fig.savefig(f'{OUT}.png', dpi=600)

fig.savefig(f'{OUT}.tiff', dpi=600, pil_kwargs={'compression': 'tiff_lzw'})

with open(f'{OUT}_peaks.csv', 'w', newline='') as fh:

w = csv.writer(fh)

w.writerow(['Spectrum', 'Nominal (cm-1)', 'Measured (cm-1)', 'Height above local line (counts)',

'Height / noise', 'Class', 'Assignment', 'Labelled'])

w.writerows(rows)

print(f'noise: EV {noise["EV"]:.1f}, NV {noise["NV"]:.1f} counts')

for r in rows:

print(r)